# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of production lines for waste recycling	p 1 /…

Publication reference: TS/2020/421-733_VM/Sup-12

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Plastic cutting machine. Polyethylene of membranes and for grinding hard thin-walled polyethylene items. The capacity of the unit, kw: 15-30. 380V/50Hz, with three-phase power supply, Rotor diameter, mm: 350-450 Rotor width, mm: 400-600 Number of blades on the rotor, unit: minimum 8 Manual top loading. Trunk size, mm, minimum 400x400 Productivity. • In the case of grinding of hard thin-walled polyethylene, kg/h: minimum 80 • In the case of crushing of polyethylene films, kg/hour: at least 30 Maximum size of polyethylene objects after cutting: 8mm Screw conveyor sand dryer With 380V/50Hz three-phase power supply. Engine power, kW: 4 - 8. Productivity, kg/h: 200-500. The maximum total electric power, kW: 35 Reducing the moisture content of the loaded sand by at least 5 % Maximum length, mm: 5.000 Maximum weight, kg: 1.50 0 Polymerase extruder-mixer with uniform intake system. Productivity, kg/h: 150-250 With 380V/50Hz three-phase power supply Maximum power , kW: 90 Minimum screw diameter, mm: 70 Maximum weight, kg: 3.000 The screw must have high wear resistance. Polymerase mixer should be able to adjust screw rotation. the extruder-mixer is regulated from the control panel, where the temperature indicators of the three zones should also be visible. Hydraulic Press Pressing force, tons: minimum 61. Engine power, kW: 4-8. 380v/50Hz, three-phase power supply, The surface of the table section of the press is at least 500x500 mm. The height of the table part of the press from the floor, mm: 700-950. The step of the latch, mm: 250-450, the step of the lower pusher, mm: 50-160. Minimum drive capacity, kN: 100 Must have removable top plate. Weight, tons: 2.8 – 4 Two-part mold. Must be made of special hardened steel and have special pushers compatible with the supplied hydraulic press. Molding time, seconds: 30-70 "Hexahedron slabs" mold (size of final product, mm: 145x35, weight, kg: 2.25) Two-part mold. It must be made of special hardened steel, which will have special pushers compatible with the supplied hydraulic press. Molding time, seconds: 30-70 Final product size: 375x215x45 mm, weight: 2.25 kg Two-part mold. Must be made of special hardened steel, have special pushers compatible with supplied hydraulic press. Molding time, seconds: 30-70 Final product size: 375x215x45 mm, weight: 2.25 kg Two-part mold. Must be made of special hardened steel, have special pushers compatible with supplied hydraulic press. Molding time, seconds: 30-70 Final product size: 240x128x45mm, weight: 2.06kg |  |  |  |